Hazardous, NON-Dangerous Goods

1. MATERIAL AND SUPPLY COMPANY IDENTIFICATION

# Product name: EGX

Recommended use: Food safe industrial lubricant and protection spray.

Supplier:; WilliamsWarn Ltd. NZBN:; 9429034306338. Street Address:; 804 Omahu Road Hastings New Zealand. Telephone:; +64 6 650 2336. Facsimile:; +64 6 650 2336.

Emergency Telephone number: +61 0417638 004

# 2. HAZARDS IDENTIFICATION

This material is hazardous according to the criteria of Safe Work Australia GHS 7.

Signal Word Danger

# Hazard Classifications

Flammable Liquids - Category 4

Aspiration Hazard - Category 1

# Hazard Statements

H227 	Combustible liquid.

H304 	May be fatal if swallowed and enters airways.

# Prevention Precautionary Statements

| P102 | Keep out of reach of children. |
| --- | --- |
| P103 | Read carefully and follow all instructions. |
| P210 | Keep away from heat, sparks, open flames, hot surfaces and other ignition sources. No smoking. |
| P280 | Wear protective gloves and protective clothing including eye protection. |

# Response Precautionary Statements

| P101 | If medical advice is needed, have product container or label at hand. |
| --- | --- |
| P301+P310 | IF SWALLOWED: Immediately call a POISON CENTRE or doctor. |
| P331 | Do NOT induce vomiting. |
| P370+P378 | In case of fire: Use alcohol-resistant foam, dry chemical or standard foam to extinguish. |

# Storage Precautionary Statements

P403 	Store in a well-ventilated place.

P405 	Store locked up.

# Disposal Precautionary Statement

P501 	Dispose of contents and container in accordance with local, regional, national and international regulations.

Poison Schedule: S5 Caution

# DANGEROUS GOOD CLASSIFICATION

Not classified as Dangerous Goods by the criteria of the "Australian Code for the Transport of Dangerous Goods by Road & Rail" and the "New Zealand NZS5433: Transport of Dangerous Goods on Land".

Classified as a C1 (COMBUSTIBLE LIQUID) for the purpose of storage and handling, in accordance with the requirements of AS 1940.  Refer to State Regulations for storage and transport requirements.

| 3. COMPOSITION INFORMATION |  |  |
| --- | --- | --- |
| CHEMICAL ENTITY | CAS NO | PROPORTION |
| Naphtha, petroleum, hydrotreated heavy | 64742-48-9 | 60 - 90 % |
| Lanolin | 8006-54-0 | 10 - 30 % |
| Ingredients determined to be non-hazardous |  | Balance ____________ 100% |
| 4. FIRST AID MEASURES |  |  |

If poisoning occurs, contact a doctor or Poisons Information Centre (Phone Australia 131 126, New Zealand 0800 764 766).

Inhalation:  Remove person from exposure - avoid becoming a casualty.  Remove contaminated clothing and loosen remaining clothing.  Allow person to assume most comfortable position and keep warm.  Keep at rest until fully recovered.  Seek medical advice if effects persist.

Skin Contact:  If skin or hair contact occurs, remove contaminated clothing and flush skin and hair with running water.  If swelling, redness, blistering or irritation occurs seek medical assistance.  For gross contamination, immediately drench with water and remove clothing. Continue to flush skin and hair with plenty of water (and soap

if material is insoluble).  For skin burns, cover with a clean, dry dressing until medical help is available.  If blistering occurs, do NOT break blisters.  If swelling, redness, blistering, or irritation occurs seek medical assistance.

Eye contact:  If in eyes wash out immediately with water. In all cases of eye contamination, it is a sensible precaution to seek medical advice.

Ingestion:  Immediately rinse mouth with water.  If swallowed, do NOT induce vomiting.  Give a glass of water to drink.  Never give anything by the mouth to an unconscious person.  If vomiting occurs give further water.  Immediately call Poisons Centre or doctor.

PPE for First Aiders:  Wear safety shoes, overalls, gloves, safety glasses.  Available information suggests that gloves made from nitrile rubber should be suitable for intermittent contact.  However, due to variations in glove construction and local conditions, the user should make a final assessment.  Always wash hands before smoking, eating, drinking or using the toilet.  Wash contaminated clothing and other protective equipment before storing or re-using.

Notes to physician:  Treat symptomatically

# 5. FIRE FIGHTING MEASURES

Hazchem Code: Not applicable

Suitable extinguishing media: If material is involved in a fire use alcohol-resistant foam, dry chemical or standard foam.

Specific hazards: Combustible liquid; will fuel a fire in progress.

Fire fighting further advice: On burning or decomposing may emit toxic fumes.  Fire fighters to wear selfcontained breathing apparatus and suitable protective clothing if risk of exposure to vapour or products of combustion or decomposition.

6. ACCIDENTAL RELEASE MEASURES

# SMALL SPILLS

Wear protective equipment to prevent skin and eye contamination.  Avoid inhalation of vapours or dust (from dried product).  Wipe up with absorbent (clean rag or paper towels).  Collect and seal in properly labelled containers or drums for disposal.

# LARGE SPILLS

If safe to do so, shut off all possible sources of ignition.  Clear area of all unprotected personnel.  Slippery when spilt.  Avoid accidents, clean up immediately.  Wear protective equipment to prevent skin and eye contamination and the inhalation of vapours.  Work up wind or increase ventilation.  Contain - prevent run off into drains and waterways.  Use absorbent (soil, sand or other inert material).  Use a spark-free shovel.  Collect and seal in properly labelled containers or drums for disposal.  If contamination of crops, sewers or waterways has occurred advise local emergency services.

Dangerous Goods - Initial Emergency Response Guide No: Not applicable

# 7. HANDLING AND STORAGE

Handling: Avoid eye contact and repeated or prolonged skin contact.  Avoid inhalation of vapour, mist or aerosols.

Storage: Store in a cool, dry, well-ventilated place and out of direct sunlight.  Store away from foodstuffs.  Store away from incompatible materials described in Section 10.  Store away from sources of heat and/or ignition.  Store locked up.  Keep container standing upright.  Keep containers closed when not in use - check regularly for leaks.

Classified as a C1 (COMBUSTIBLE LIQUID) for the purpose of storage and handling, in accordance with the requirements of AS 1940.  Refer to State Regulations for storage and transport requirements.

This material is a Scheduled Poison Schedule 5 (Caution) and must be stored, maintained and used in accordance with the relevant regulations.

# 8. EXPOSURE CONTROLS / PERSONAL PROTECTION

National occupational exposure limits: No value assigned for this specific material by Safe Work Australia.

Biological Limit Values: As per the "National Model Regulations for the Control of Workplace Hazardous Substances (Safe Work Australia)" the ingredients in this material do not have a Biological Limit Allocated.

Engineering Measures: Natural ventilation should be adequate under normal use conditions.

Personal Protection Equipment: SAFETY SHOES, OVERALLS, GLOVES, SAFETY GLASSES

Personal protective equipment (PPE) must be suitable for the nature of the work and any hazard associated with the work as identified by the risk assessment conducted.

Wear safety shoes, overalls, gloves, safety glasses.  Available information suggests that gloves made from nitrile rubber should be suitable for intermittent contact.  However, due to variations in glove construction and local conditions, the user should make a final assessment.  Always wash hands before smoking, eating, drinking or using the toilet.  Wash contaminated clothing and other protective equipment before storing or re-using.

Hygiene measures:  Keep away from food, drink and animal feeding stuffs.  When using do not eat, drink or smoke.  Wash hands prior to eating, drinking or smoking.  Avoid contact with clothing.  Avoid eye contact and repeated or prolonged skin contact.  Avoid inhalation of vapour, mist or aerosols.  Ensure that eyewash stations and safety showers are close to the workstation location.

| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |
| --- | --- | --- |
| Form: Colour: | Liquid Brown |  |
| Solubility: |  | Immiscible in water |
| Density: |  | 0.80 - 0.84 g/ml |
| Vapour Pressure: |  | N Av |
| Flash Point (°C): |  | >75ºC |
| Autoignition Temperature (°C): |  | >250ºC |
| Boiling Point/Range (°C): |  | >150ºC |
| pH: |  | N Av |
| Viscosity: |  | N Av |
| 10. STABILITY AND REACTIVITY |  |  |

Chemical stability: This material is thermally stable when stored and used as directed.

Conditions to avoid: Elevated temperatures, naked flames and sources of ignition

# Incompatible materials: Strong oxidising agents

Hazardous decomposition products: Carbon dioxide, carbon monoxide and other organic complexes upon incomplete burning or oxidation

Hazardous reactions: No information available

# 11. TOXICOLOGICAL INFORMATION

No adverse health effects expected if the product is handled in accordance with this Safety Data Sheet and the product label.  Symptoms or effects that may arise if the product is mishandled and overexposure occurs are:

# Acute Effects

Inhalation: Material may be an irritant to mucous membranes and respiratory tract.

Skin contact: Contact with skin may result in irritation. Product may also have an emollient (moisturising) effect on the skin.

Ingestion: Swallowing can result in nausea, vomiting and irritation of the gastrointestinal tract.  May cause lung damage if swallowed.  Small amounts of liquid aspirated into the respiratory system during ingestion or vomiting may cause bronchopneumonia or pulmonary oedema.

Eye contact: May be an eye irritant. Contact is unlikely to result in permanent damage.

# Acute toxicity

Inhalation: This material has been classified as not hazardous for acute inhalation exposure.  Acute toxicity estimate (based on ingredients): LC50 > 20.0 mg/L for vapours or LC50 > 5.0 mg/L for dust and mist.

Skin contact: This material has been classified as not hazardous for acute dermal exposure.  Acute toxicity estimate (based on ingredients): LD50 > 2,000 mg/Kg bw

Ingestion: This material has been classified as not hazardous for acute ingestion exposure.  Acute toxicity estimate (based on ingredients): LD50 > 2,000 mg/Kg bw

Corrosion/Irritancy:   Eye: this material has been classified as not corrosive or irritating to eyes.  Skin: this material has been classified as not corrosive or irritating to skin.

Sensitisation: Inhalation: this material has been classified as not a respiratory sensitiser.  Skin: this material has been classified as not a skin sensitiser.

Aspiration hazard: This material has been classified as Aspiration Hazard - Category 1. May be fatal if swallowed and enters airways.

Specific target organ toxicity (single exposure): This material has been classified as not a specific hazard to target organs by a single exposure.

# Chronic Toxicity

Mutagenicity: This material has been classified as not a mutagen.

Carcinogenicity: This material has been classified as not a carcinogen.

Reproductive toxicity (including via lactation): This material has been classified as not a reproductive toxicant.

Specific target organ toxicity (repeat exposure): This material has been classified as not a specific hazard to target organs by repeat exposure.

# 12. ECOLOGICAL INFORMATION

Avoid contaminating waterways.

Acute aquatic hazard: This material has been classified as not hazardous for acute aquatic exposure.

Long-term aquatic hazard: This material has been classified as not hazardous for chronic aquatic exposure.

Ecotoxicity: No information available

Persistence and degradability: This product contains an animal fat that is considered biodegradable.

Bioaccumulative potential: Does not bioaccumulate

Mobility: This product is unlikely to be mobile on release to the environment.

# 13. DISPOSAL CONSIDERATIONS

Persons conducting disposal, recycling or reclamation activities should ensure that appropriate personal protection equipment is used, see "Section 8. Exposure Controls and Personal Protection" of this SDS.

If possible, material and its container should be recycled.  If material or container cannot be recycled, dispose in accordance with local, regional, national and international Regulations.

14. TRANSPORT INFORMATION

# ROAD AND RAIL TRANSPORT

Not classified as Dangerous Goods by the criteria of the "Australian Code for the Transport of Dangerous Goods by Road & Rail" and the "New Zealand NZS5433: Transport of Dangerous Goods on Land".

# MARINE TRANSPORT

Not classified as Dangerous Goods by the criteria of the International Maritime Dangerous Goods Code (IMDG Code) for transport by sea.

# AIR TRANSPORT

Not classified as Dangerous Goods by the criteria of the International Air Transport Association (IATA) Dangerous Goods Regulations for transport by air.

# 15. REGULATORY INFORMATION

This material/constituent(s) is covered by the following requirements:

The Standard for the Uniform Scheduling of Medicines and Poisons (SUSMP) established under the Therapeutic

Goods Act (Commonwealth): S5 Caution

AICIS Status: All components of this product are listed on or exempt from the Australian Inventory of Industrial Chemicals (AIIC).

# 16. OTHER INFORMATION

Reason for issue: 	Revised

Supersedes: 	NA

Controlled Document: LAN-0010 SDS-EGX

This information was prepared in good faith from the best information available at the time of issue. It is based on the present level of research and to this extent we believe it is accurate. However, no guarantee of accuracy is made or implied and since conditions of use are beyond our control, all information relevant to usage is offered without warranty. The manufacturer will not be held responsible for any unauthorised use of this information or for any modified or altered versions.

If you are an employer, it is your duty to tell your employees, and any others that may be affected, of any hazards described in this sheet and of any precautions that should be taken.

Safety Data Sheets are updated frequently. Please ensure you have a current copy.